Instruction Manual

Selectable Microstep

5-ph Stepping Motor Driver

##### RD-053MS

RORZE CORPORATION

Be sure to read this instruction before using the product.

This section describes the safety precautions to observe to prevent danger to you or others and damage

to your property in advance.

Instructions before use

This product is a device to be required high safety and reliance. It isn’t developed to use in the devices having danger that its failure or malfunction threatens human lives directly or harms human body (Aerospace machines, disaster and crime prevention machines, all kinds of safety devices, etc.). If you use this product in a device for general purpose, but establish the protection function, etc. and look to your own safety, also since testing the safety fully, please warrant it as a device on the part of you. If you use this product in a device like the above, please inform our company. RORZE will not be responsible for any damage incurred by using without obtaining permission from our company in advance.

！

WARNING

Indicates potential danger. Used to indicate that neglecting the procedures or instructions will lead to conditions that may result in death or severe injury. - Do not use near the flammable materials, where water might be splashed, combustibles because of the danger of injure or fire. - Do not move or wire etc. in operating because of the danger of electric shock or injure. - Do not bend, pull or nip lead wires forcibly because of the danger of electric shock, fire, or failure - Do not use damaged lead wire because of the danger of electric shock, fire, or failure. - Please wire certainly not to be a bad wiring or tightening because of the danger of electric shock, fire, and failure. - Do not touch inside this product because of the danger of electric shock, and failure. - Do not disassemble or change this product because of the danger of electric shock and failure. - Do not wire or operate with wet hands because of the danger of electric shock. - Any works(transportation, setting, wiring, operation, maintenance, inspection) must be performed only by the personnel who has expert knowledge because of the danger of electric shock, injure and fire.

！

CAUTION

Indicates potential danger. Used to indicate that neglecting the procedures or instructions may cause medium or slight injury or damage to instruments or equipment. ◇Please verify the product is what you ordered. Setting the wrong goods causes fire and failure. Do not turn on the power until verifying the followings; - Do not use power supply except the one outputs 18～40ＶDC. - Operate the power supply within the specified input/output current and input/output voltage limits only. - Do not wire and short-circuit each input/output terminal in wrong. - Do not use with the motor except the stepping motor. - Operate the rated current of stepping motor within the specified input current limits only. - Because this product generates heat, please make it stick to metal board etc. or put the fan and radiate enough. - When connecting with terminal, use a screwdriver whose tip fits an adjustment slot. Tighten the screw in the torque of less than 3.5kgf･cm(0.35N･m)(proper torque is 2.5kgf･cm(0.25N･m)) - When connecting to the device and starting to operate, please begin with the condition that can do emergency stop any time. If you don’t follow the above, it causes fire and failure. - When the extraordinary noise has occurred, immediately turn off the power supply. Otherwise, There is danger of injure and fire. - In operation or right after motor is stationary, do not touch this product. There is danger of burn. - Do not move this product while holding terminal or lead wire. There is danger of injure as a result of being dropped. - Do not put this product on unstable place or where it’s easy to fall down. There is danger of injure as a result of being dropped.

- Also, even the matters described in this caution may result in serious result (death or severe injury) depending on usage.  Please follow this instruction because it indicates important meaning.

- Table of Contents

- Introduction	１

１．Description	１

２．Features	１

３．Specifications	２

４．Part Name	２

５．Current Adjustment	３

5-1 Run Current Adjustment	３

5-2 Stop Current Adjustment	３

６．Explanation of Terminals	４

6-1 Clock Inputs (CW/CLK, CCW/UD)	４

6-2 Full Step Input (4P IN +/-)	５

6-3 Free Input (FREE +/-)	５

6-4 Phase Output (PHASE +/-)	５

6-5 Alarm Output (ALARM +/-)	５

７．Explanation of ALARM LED	５

８．Dip Switches	６

8-1 Current Range Switch (3A/1.5A)	６

8-2 Clock Input Switch (1CK/2CK)	６

8-3 Auto. Current Down Switch (C.DOWN/OFF)	６

8-4 Microstep Selection Table (D1～D5)	６

９．Timing Diagrams	７

１０．Input/Output Circuits	８

10-1 Clock Inputs (CW/CLK, CCW/UD)	８

10-2 Other Input Circuits (4P IN +/-, FREE +/-)	８

10-3 Output Circuits (PHASE +/-, ALARM +/-)	８

１１．Wiring Diagram	９

11-1 Motor Selection	９

１２．Radiation	１０

１３．Other Functions	１０

13-1 Auto. current down	１０

13-2 Over heating protection circuit	１０

13-3 Over current protection circuit	１０

13-4 Low voltage protection circuit	１０

１４．Power Supply Requirement	１０

１５．Relationship between Frequency(pps) and Motor speed(rpm)	１１

１６．Dimensions	１１

- INSTRUCTION MANUAL FOR RD-053MS

- INTRODUCTION

Thank you for purchasing a RORZE MicroMate product.  This new breakthrough in ideas and technology from RORZE has brought you an easy-to-use, cost-effective, selectable micro driver.

We hope you will be completely satisfied with your purchase and that it will greatly enhance your automation capabilities.

Please read this manual carefully before installing your MicroMate driver to get the most from your application.  As your motion control needs increase, RORZE has an ever-increasing number and type of Controllers and Drivers that meet those needs.  Keep us in mind for your future automation endeavors.

If you have any questions regarding this product or an application that you would like to discuss with us, direct your questions to :

RORZE Corporation,

Control Device Division, Sales Section

１．Description

- The RD-053MS provides the solution to all of the above difficulties through a superior design and state-of-the-art hybrid technology.  Due to miniaturization and power efficiency, pulse generator and translator are combined into a single compact package which can be installed right at the step motor.  Also, by utilizing a DC power supply of 24V or 36V (as is common to solenoid valves, etc.), several units can be connected to a simple programmable controller or personal computer with great economy of wiring.

The RD-053MS is a high resolution step motor driver featuring selectable microstep.  Microstep drive is the drive method to divide a basic step angle of motor by controlling the current applying to the motor.  This driver can divide a step angle from 1～400 microsteps/step.

In case of 0.72ﾟ motor, the microstep resolution at full step driving is 500 resolution/step(0.72ﾟ/pulse), but if you select 400 resolution at microstep driving, you can make the microstep resolution 200,000 resolution/step(0.0018ﾟ/ pulse).  Microstep can come true super low speed drive or exact positioning.

Also, at full step driving, there are problems, vibration or resonance, but changing to the microstep driving make the vibration decrease.

- ２．Features

･ High power (3A/phase max.)

･ Supply voltage from 18～40VDC

･ Selectable microstep (22 selections) using the dip switch

･ Isolated inputs and outputs (photo-coupler)

･ Circuit protections to protect against overheating, low voltage and over current

of motor connection.

･ Can change the arrangement range of motor drive current using the switch

(max. 1.5A or max. 3A)

･ Selectable clock – 1clk. or 2clk. Input

･ Auto. current down (Adjustable stop current)

- ３．Specifications

| Supply voltage | Single 18～40VDC(including ripple) |
| --- | --- |
| Supply current | 2 times rated coil current per phase |
| Motor current | 0.5～3A/phase (Can change current by Run Current Pot) |
| Drive method | Bipolar, constant current chopper method |
| Excitation method | Microstep |
| Microstep resolution | Up to 400 microsteps/step |
| Position repeatability | ±1 microstep with no load in one direction (with 50 microsteps/step) |
| Auto. current down | 0～80% of the rated current after about 0.3 seconds of inactivity (Setting at shipment is 50%) Even if the motor rotation is stationary, the auto. current down doesn’t work under the condition applying voltage between clock input terminals or turning auto. current down function OFF by dip switch. |
| Protective circuitry | Overheat, over current and low voltage protection |
| Response frequency | 500 kpps Max. |
| Weight | Approx. 580g (20.5oz.) |
| Outside dimensions | 63H x 56W x 105Dmm (2.5”H x 2.2”W x 4.1”D) |

- ４．Part Name

RUN CURRENT POT       STOP CURRENT POT

CLOCK INPUT

FULL STEP INPUT

FREE INPUT

PHASE OUTPUT

ALARM OUTPUT

ARARM LED

MOTOR OUTPUT

POWER SUPPLY

- Fig.1. View of Terminals

５．Current Adjustment

CURRENT

0       10 0      10

RUN     STOP

Fig.2. Current Setting Pots

5-1 Run Current Adjustment

！

- Caution

Please use after adjust the current within the rated current of stepping motor. Exceeding the rated current causes the failure of motor or fire.

Set the current range using the dip switch. To adjust the stop and run current, use the respective pots.  (Setting at shipment is 0A.)

Follow the graphs below to get the approximate division on the pot.

The value of the setting will be ±10 % of the set value.

If the drive current is less than 1.5A, you had better set the dip switch to 1.5A side, because it makes current Adjustment easy.

Dip switch

3A

1.5A

- Fig.3. Run Current Setting

5-2 Stop Current Adjustment

- You can set the stop current to any value between 0～80% of the run current by adjusting Stop Current Pot. (Setting at shipment is 50 %)

Fig.4. Stop Current Setting

６．Explanation of Terminals

6-1 Clock Inputs (CW/CLK, CCW/UD)

！

- Caution

Please set the current between clock input terminals in the range of 8 to 20mA. If the current is over 20mA, the driver will be broken. If the current is less than 8mA, it becomes the cause of malfunction.

- In case of using Two Clock Input (2CK)

- (Input 2 kinds of signals - CW clock pulse and CCW clock pulse)

- CW+/-

- Motor rotates in CW direction with a pulse current of 8～20mA

from CW+ to CW- terminal.

- CCW+/-

- Motor rotates in CCW direction with a pulse current of 8～20mA

from CCW+ to CCW- terminal.

- In case of using One Clock Input (1CK)

- (Input clock pulse signal and rotation direction signal.(CW, CCW))

- CLK+/-

- Motor rotates in CW direction with a pulse current of 8～20mA

from CLK+ to CLK- terminal and UD input off.

UD+/-

- Motor rotates in CCW direction with a pulse current of 8～20mA

from CLK+ to CLK- terminal and UD input turned ON.

- The current in case of using at 4.5～5.5V is 10～15mA.

CCW       CW

Fig.5. Direction of Rotation

6-2 Full Step Input (4P IN +/-)

Turning 4P IN input ON will make the motor operate in full step mode.  If this 2P IN input is OFF, the motor will run in microstep mode.

6-3 Free Input (FREE +/-)

Turning FREE input On will turn OFF the current to the motor allowing free motion without holding current.

NOTE:  Do not turn this input ON when motor is running at high speed, this operation would damage the amplifier circuits.

6-4 Phase Output (PHASE +/-)

When excitation of coil stands by at origin phase, PHASE output will be turned ON.

This output become ON every time the motor move 7.2゜in case of 0.72゜motor and 3.6゜in case of 0.36゜motor.

However, duty of Output ON is 10%.

6-5 Alarm Output (ALARM +/-)

Overheating protection circuit intervenes turning ALARM output ON.  (Open collector output ON)

７．Explanation of ALARM LED

Overheating protection circuit intervenes turning ALARM LED ON.

- ８．Dip Switches

0-1

3A         1.5A

1CK         2CK

C.DOWN         OFF

D1

D2

D3

D4

D5

Fig.6. Dip switches

8-1 Current Range Switch (3A/1.5A)

When you set RUN CURRENT ADJUSTMENT at max. value,

switch the motor current per phase. (See Fig.3)

8-2 Clock Input Switch (1CK/2CK)

- Change pulse configuration to Two clock input (2CK) or

- One clock input (1CK).

8-3 Auto. Current Down Switch (C.DOWN/OFF)

- Setting this OFF, Auto. Current Down doesn’t work.

8-4 Microstep Selection Table (D1～D5)

！

- Caution

Do not set the dip switchs except the below table. It causes malfunction or failure.

- Can select step angle from among 22 sorts using dip switch D1～D5.

1

Step angle = Motor basic step angle × ───────────

Microstep resolution

Dip switch setting table:

| D5 D4 D3 D2 D1 | Microstep Resolution(M) |  | D5 D4 D3 D2 D1 | Microstep Resolution(M) |
| --- | --- | --- | --- | --- |
| ０ ０ ０ ０ １ | ４００ |  | ０ １ ０ ０ ０ | ６４ |
| ０ ０ ０ １ ０ | ２００ |  | ０ １ ０ ０ １ | ３２ |
| ０ ０ ０ １ １ | １００ |  | ０ １ ０ １ ０ | １６ |
| ０ ０ １ ０ ０ | ５０ |  | ０ １ ０ １ １ | ８ |
| ０ ０ １ ０ １ | ２５ |  | ０ １ １ ０ ０ | ４ |
| ０ ０ １ １ ０ | １２.5 |  | ０ １ １ ０ １ | ２ |
| ０ ０ １ １ １ | ６.25 |  | ０ １ １ １ ０ | １ |

| D5 D4 D3 D2 D1 | Microstep Resolution(M) |
| --- | --- |
| １ ０ ０ ０ ０ | ３２０ |
| １ ０ ０ ０ １ | １６０ |
| １ ０ ０ １ ０ | ８０ |
| １ ０ ０ １ １ | ４０ |
| １ ０ １ ０ ０ | ２０ |
| １ ０ １ ０ １ | １０ |
| １ ０ １ １ ０ | ５ |
| １ ０ １ １ １ | ２.5 |

- ９．Timing Diagrams

- Switching Direction of Rotation

Two clock input:

２ＣＫ

Ｔ1＞５μS

Ｔ1

Ｔ1

１ＣＫ

Ｔ2＞２μS

Ｔ2

Ｔ3

Ｔ3＞100μS

ＣＷ

ＣＣＷ

ＣＬＫ

ＣＬＫ

ＵＤ

4P ＩＮ

(ＣＷ・ＣＣＷ)

Fig.7. Two Clock Timing Diagram

One clock input:

- Fig.8. One Clock Timing Diagram

Switching between microstep and full step or vice versa

- Fig.9. Micro/Full step Switching Timing Diagram

- Note1: The above diagrams indicate “－” terminal voltage wave form (minus logic) in case of

- connecting external power supply to each “＋” terminal, and open collector output of

- external controller to “－” terminal.

Note2: The pulse count will not be lost as long as parameters T1～T3 are within the spec.

Note3: Motor will rotate 1 step at the rising edge of pulse(CW･CCW･CLK).

(At the timing clock switches from ON to OFF)

- １０．Input/Output Circuits

！

- Caution

Do not exceed max. rated current･voltage of each I/O circuit. It causes failure or malfunction.

270Ω

Inside

8～20mA

ＣＬＫ／ＣＷ

ＵＤ／ＣＣＷ

＋

－

Photo-coupler

1kΩ

10-1 Clock Inputs (CW/CLK, CCW/UD)

- Fig.10. Clock Inputs

Please use in the 8～20mA signal current.
(When input signal voltage is 4.5～5.5V, the signal current is 10～15mA.)

When the input signal voltage is high(24V or so) and connecting directly makes the current over 20mA, please insert a current limiting resistor in series and adjust the signal current 8～20mA.

more than 1μS

less than 1μS

4.5～5.5V

0V

- Fig.11. Pulse Specification

4Ｐ ＩＮ

ＦＲＥＥ

＋

－

Inside

1kΩ

Photo-coupler

2～10mA

10-2 Other Input Circuits (4P IN +/-, FREE +/-)

- Fig.12. Input circuits

Please use in the 2～10mA signal current.

(When the input signal voltage is 5V, the current is 3.8mA.)

When the input signal voltage is high (24V or so) and connecting directly makes the current over 10mA, please insert a current limiting resistor in series and adjust the current 2～10mA.

10-3 Output Circuits (PHASE +/-, ALARM +/-)

ＰＨＡＳＥ

ＡＬＡＲＭ

＋

－

2.2kΩ

30V/100mA Max.

Inside

Photo-coupler

- Fig.13. Output circuits

Do not apply the voltage more than 30V, the current more than 100mA between terminals.

- １１．Wiring Diagram

！

- Caution

Do not put on the power before verify whether there are not mis-wiring and short-circuit and make the driver connect certainly. There is danger of the fire or failure. Please tighten the terminals at the torque levels less than 3.5kgf･cm (0.35N･m).

Use AWG 20 or less (greater than 0.5mm２) wire for connecting the driver to the motor and use twisted wire or shielded wire pair for the signal input wiring.  Please tighten the terminals less than 3.5kgf･cm (0.35N･m). (Proper torque is 2.5kgf･cm (0.25N･m))

18～40VDC

GND

Controller

+5V

Ｍ

A

B

C

D

E

10 LEAD WIRE MOTOR

- Fig.14. Wiring Diagram

- 11-1 Motor Selection

This driver is suitable for every stepping motor which type is HB or PM, rated current per phase is 0.5～3A/phase and rated voltage is less than supply voltage × 0.7(V).

RORZE 5-Ph Stepping Motors (Torque -- 1kgf･cm = 13.9oz･in   Inertia -- 1g･c㎡ = 5.46745×10-3oz･in2)

| ModelNo. | Holding Torque |  | FullStep Angle (Degree) | Rated Current (A/ph) | Rotor Inertia (g･c㎡) | Resistance （Ω） | Inductance (mH) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kgf･cm) | (N･m) |  |  |  |  |  |
| RM5407SM/DM | 0.75 | 0.074 | 0.36 | 1.5 | 13.2 | 0.6 | 0.45 |
| RM5411SM/DM | 1.1 | 0.11 | 0.36 | 1.5 | 24 | 0.65 | 0.93 |
| RM5414SM/DM | 1.4 | 0.14 | 0.36 | 1.5 | 36 | 0.89 | 0.87 |
| RM5623S/D | 2.3 | 0.23 | 0.72 | 3.0 | 57 | 0.19 | 0.25 |
| RM5640S/D | 4.0 | 0.39 | 0.72 | 3.0 | 105 | 0.24 | 0.37 |
| RM5685S/D | 8.0 | 0.78 | 0.72 | 3.0 | 235 | 0.46 | 1.16 |
| RM59A2S/D | 12.5 | 1.23 | 0.72 | 3.0 | 520 | 0.33 | 2.0 |
| RM59B2S/D | 22.0 | 2.2 | 0.72 | 3.0 | 1200 | 0.36 | 2.5 |
| RM59D0S/D | 40.0 | 3.9 | 0.72 | 3.0 | 1800 | 0.66 | 5.4 |

Color of RORZE motors wire

|  | Terminal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | A | B | B | C | C | D | D | E | E |
| RORZE MOTOR | Blue | Red | White | Yellow | Brown | Purple | Black | Gray | Orange | Green |

- １２．Radiation

！

- Caution

Please radiate drivers and motors enough. If it is insufficient, temperature rise causes malfunction, failure or fire.

Never allow the motor temperature to rise above 100℃ and driver temperature should not exceed 70℃.  To dissipate heat, fix a radiator board, fan, etc.

- １３．Other Functions

13-1 Auto. current down

After about 0.3 seconds of inactivity, current will be reduced to the preselected percentage of running current. This reduces driver and motor heat generation during idle periods.

Even if the motor rotation is stationary, the auto. current down doesn’t act under the condition applying the voltage between clock input terminals or auto. current down function is turned OFF using dip switch.

13-2 Over heating protection circuit

This acts when the temperature inside the driver raises abnormally.  It turns ALARM Output ON and ALARM LED turns ON.  Furthermore, if the temperature keeps on raising, it stops motor compulsively and works Auto. Current down to prevent the driver burn out.  If the body temperature drops about 10℃ below the triggered temperature, returns automatically.

- 13-3 Over current protection circuit

This works when detects abnormal current inside the driver by mis-wiring of motor or short etc.

13-4 Low voltage protection circuit

The driver has a built-in low voltage protection circuit to prevent current overload.  The low supply voltage condition normally occurs when power is turned ON.

- １４．Power Supply Requirement

The supply current required for driver and motor varies, depending on the supply voltage and pulse (clock) frequency and motor’s inductance and rated current and holding torque.  Also, the ripple according to the cycle of PWM (50kHz) and RPM is added into the supply current.  Please use the power supply which current is more than 2 times the rated current of the motor as a standard.

If other devices share the same power supply and voltage change can’t be allowed, then use the power supply which can flow 1.7 times of the max value of the supply current, or incorporate a large capacitor.

１５．Relationship between Frequency(pps) and Motor speed(rpm)

The number of pulses per second is the frequency, and the unit of measure is PPS.

Further, the motor speed can be derived from the frequency using the following formula:

Step Angle／Microstep resolution(M)×Frequency×60

Motor speed(rpm)＝─────────────────────────

360(degree)

Example

Step Angle: 0.72degree,  Microstep resolution(M): 10,  Frequency: 10,000pps

0.72／10×10,000×60

Motor speed(rpm)＝───────────＝ 120 rpm

360

- １６．Dimensions

Fig.15. Dimensions (not to scale)